Standard Operating Procedure

PHS - Template

This is an SOP template and is not complete until: 1) lab specific information is entered into the box below 2) lab specific protocol/procedure is added to the protocol/procedure section 
3) SOP has been signed and dated by the PI and relevant lab personnel.

Print a copy and insert into your Lab Safety Plan.

Section 1 – Lab-Specific Information

| Building/Room(s) covered by this SOP: | Click here to enter text. |
| --- | --- |
| Department: | Click here to enter text. |
| Principal Investigator Name: | Click here to enter text. |
| Principal Investigator Signature: |  |

Section 2 – Hazards

Particularly Hazardous Substances (PHSs) are defined by OSHA as ““select carcinogens,” reproductive toxins and substances which have a high degree of acute toxicity.” Below are the PHSs to be used in this lab, along with what category of hazard they represent for the purposes of being listed (carcinogen, reproductive hazard, acutely toxic). Note: There may be other hazards associated with the chemicals listed below, see SDS for each specific PHS for more information.

Carcinogens:

Reproductive Hazards:

Acutely Toxic:

[Choose which GHS or other hazard pictogram is applicable for the chemicals described above]

Section 3 – Engineering Controls and Personal Protective Equipment (PPE)

[See Section 8 of the chemical’s SDS for additional relevant information.]

Engineering Controls/Containment Devices: Please list here what engineering controls will be used to contain the PHSs to be used.

[e.g. “All carcinogens, reproductive hazards, and acutely toxic substances listed above will be used in a fume hood except for _____, which will be utilized in the general lab area following appropriate containment and handling procedures outlined in Section 7 below]

Hygiene Measures: Avoid contact with skin, eyes, and clothing. Wash hands before breaks and immediately after handling the product.

Hand Protection: Nitrile gloves should be worn during general use of the product. [List what sort of hand protection is necessary, nitrile gloves, butyl gloves, double-gloving, etc.] NOTE: Consult with your preferred glove manufacturer to ensure that the gloves you plan on using are compatible with the specific chemical being used.

Eye Protection: Safety glasses are to be worn during general use, but safety goggles should be worn if there is a risk of splashing while mixing a solution. [List what eye protection is required. ANSI approved safety glasses are minimum lab PPE; this does not include prescription glasses unless they are ANSI rated lenses and include side-shields. Chemical splash goggles are recommended when splash risk is likely or when working with concentrated corrosives. Face shields are recommended in addition to eye protection when necessary.]

Skin and Body Protection: Personnel must wear full length pants, or equivalent, and close-toed shoes. Full length pants and close-toed shoes must be worn at all times by all individuals that are occupying the laboratory area. The area of skin between the shoe and ankle must not be exposed. Laboratory coats are also minimum PPE for working with PHSs and must be appropriately sized for the individual and buttoned to their full length. [List what other skin and body protection will be needed when working with PHSs.]

Respiratory Protection: Fume hoods should be utilized as a first measure of protection. [List alternative/additional respiratory protection measures here, if needed. If personal respiratory protection must be utilized, (N-95 masks, half or full-face respirator, PAPR, SCBA, etc., contact EHS to enroll in the respiratory protection program.]

Section 4 – Special Handling, Storage Requirements and Establishment of a Designated Area

A designated area for use of PHSs must be established. The designated area can be a particular location such as a fume hood or glovebox, or it can be another portion of the lab or the entire room. Please list the location of the designated area for the PHSs to be utilized in the lab:

[e.g. “The designated area for the PHSs for this lab shall be the {entire lab, fume hood, glovebox, other location}]

Other general notes regarding PHSs

- Avoid contact with skin, eyes, and clothing.

- Keep container tightly closed in a dry and well-ventilated area. Store in original container away from direct sunlight.

- [Insert any major chemical incompatibilities here].

- Do not store in the top-most shelf of the storage cabinet. In general, do not store chemicals at or above eye level.

- [Insert any additional special handling or storage requirements here].

Section 5 – Spill and Accident Procedures

Any spills should be cleaned up by lab personnel following guidelines outline in the University Chemical Hygiene Plan. [List any other lab-specific spill and accident procedures here. See Section 6 of each PHS’s SDS for specific information.] If there is an imminent threat of a fire, pull the nearest fire alarm station to evacuate the building and dial 911. If personnel have become exposed and need medical assistance, dial 911. If the spill is minor and does not pose a threat to personnel, contact EHS at (208) 885-6524 during normal business hours (Monday – Friday, 8 AM – 5 PM) for spill cleanup assistance (dial 911 if spill occurs after hours and assistance is needed).

Section 6 – Waste Disposal Procedures

Store hazardous waste in closed containers that are properly labeled and in a designated area e.g. the lab Satellite Accumulation Area (SAA), marked with an SAA sign. [Insert waste segregation information here.] Complete an online Chemical Waste Pickup Request to arrange for disposal by EHS.

Section 7 – Protocol (Add lab specific Protocol/Procedure here)

[Insert information about how your lab will specifically handle any PHS]

NOTE: Any deviation from this SOP requires approval from PI.

Section 8 – Documentation of Training

Prior to conducting any work with PHSs, the Principal Investigator must ensure that all laboratory personnel receive training on the content of this SOP.

I have read and understand the content of this SOP: (signature of all users is required)

| Name | Signature | Date |
| --- | --- | --- |
| Click here to enter text. |  | Click here to enter a date. |
| Click here to enter text. |  | Click here to enter a date. |
| Click here to enter text. |  | Click here to enter a date. |
| Click here to enter text. |  | Click here to enter a date. |
| Click here to enter text. |  | Click here to enter a date. |
| Click here to enter text. |  | Click here to enter a date. |
| Click here to enter text. |  | Click here to enter a date. |
| Click here to enter text. |  | Click here to enter a date. |